Homework – Representations, Poles and Zeros

Below is a free-body diagram of the damped compound pendulum, dimensions and experimental data plotting the time response from free fall.

| L | Bar length | 0.495 | m |
| --- | --- | --- | --- |
| d | Pivot to CG distance | 0.023 | m |
| mL | Mass of pendulum | 0.43 | kg |

- If cis the viscous damping coefficient, show that the equation of motion for the damped compound pendulum sketch above is given the equation below (10 points)

θ+cJθ+mLgdLθ=0

Given the following block diagram and values, show that the open-loop transfer function is given by (5 points)

Θ(s)V(s)=1.89s2+0.039s+10.77=Gol(s)

| J | = 0.0090 | kg⋅m2 |
| --- | --- | --- |
| c | = 0.00035 | Nmsrad |
| mL | = 0.43 | kg |
| d | = 0.023 | m |
| g | = 9.81 | ms2 |
| Km | = 0.017 | NmV |

- Show that the time constant Tc=0.01475secand settling time ts=271.2secfor the damped compound pendulum given that ζ=0.0059and natural frequencyωn=2.50rads (5 points)

- Show that the complex poles s1,2=−2.4±j2.4will yield ωn=3.39radsand phase angle θ=0.785rad(5 points)