Homework: Modeling and System Identification

Below is a free-body diagram of the damped compound pendulum, dimensions and experimental data plotting the time response from free fall.

| L | Bar length | 0.495 | m |
| --- | --- | --- | --- |
| d | Pivot to CG distance | 0.023 | m |
| mL | Mass of pendulum | 0.43 | kg |

- Prove the moment of inertia Jabout the pivot point is 0.0090 kg⋅m2.  Assume a slender rigid bar and use the parallel axis theorem (5 points)

- Use the experimental data above to show that ζ=0.0059and ωn=2.50rad/sec. (5 points)

- The equation for the experimental data is given by θ+2ςωnθ+ωn2θ=0.  Compare this to equation of motion θ+cJθ+mLgdJθ=0 to show that c=0.00035N⋅m⋅srad and ωn=3.28rads (5 points)

Include an Excel plot of the data you acquired in Lab (where you used the encoder to capture angle data).  Label consecutive peaks and period.  Show calculations for ζand ωn (20 points)

θ+cJθ+mLgdJθ=KmJV

- Given the following differential equation

Use state variables x1=θ and x2=θ to show that the state space realization is (5 points)

x1x2=01−mLgdJ−cJx1x2+0KmJuy=10x1x2+00u