Homework: Pole Placement and PID Simulation

- In Lab 5, you designed a pole placement controller for the following system

Θ(s)V(s)=1.89s2+0.039s+10.77=Gol(s)

Create a pole placement controller in Simulink so that the rise time ts=3.0sec with dampingζ=1.0.  Submit and/or answer the following:

- Simulink block diagram (use screen capture) (5 points)

- Printout of Matlab code for pole and gain calculations (5 points)

- Screen capture displaying the resulting poles and gains from Matlab code (5 points)

- Scope output hardcopy (angle versus time response) to a 2.0 Volt step input (5 points)

Givenk1GcpHr=10.823s2+0.039s+10.77 where Gcprepresents the compound pendulum block, the following is a block diagram of PD control:

Where recall that:

| L | Bar length | 0.495 | m |
| --- | --- | --- | --- |
| d | Pivot to CG distance | 0.023 | m |
| mL | Mass of pendulum | 0.43 | kg |

| J | Moment of Inertia | 0.0090 | kgm2 |
| --- | --- | --- | --- |
| kgm2 | Viscous damping | 0.00035 | Nmsrad |

A. Show that the closed-loop transfer function is given by (10 points)

ΘdΘr=10.823Kp+Kdss2+0.039+10.823Kds+10.77+10.823Kp=Gcl

- Look up the Final Value Theorem and state what it is (5 points)

- Use the Final Value Theorem to show θd(∞)≈20when Kpis large and a step input Θr(s)=20s is applied to the system in Question A above (5 points)

- Provide a hard copy printout of the scope output using gainsKp=0.25, KD=0.5 (5 points)

- Provide a set of gains so that the settling time is less than 3.0 seconds and no overshoot.  Include a hard copy printout of the scope output at these gains (5 points)