Homework (Solutions)

- Fill in the blanks for the following (20%)

  - Cams turn rotary motion into an upward and downward motion

  - Throw is difference between a cam’s minimum and maximum radii

  - Eccentricity means to be off-center

  - Unlike cranks, cams have the ability to store information

  - Cams are the mechanical version of computer programs

  - A bell crank converts up-down motion to side-to-side motion

  - A 3-bar (linkage) is for straight-line motion

  - A slider-crank with a low bearing increases lever sway

  - Levers move in a circular arc

  - A spring can be used to keep the lever against the cam so it follows the profile

  - Springs in cam-followers increase friction so tensioners are used

- Answer the following (10%)

  - List, sketch and give examples of the 4 types of springs

Solution:

Compression spring: shock absorbers

Extension spring: springs in garage door rails

Torsion spring: mousetrap spring

Radial Spring: clamps and seals

  - Explain how cams examples of memory and storage

Solution:

- Cams have ability to store information

- As cam rotates, info retrieved by cam follower

- The cam follower tracks the cam’s profile and hence reproduced the same movement every cam cycle

- A series of cams on a single shaft can carry out a complex program

- Define (1 to 2 sentences), describe (1 to 3 paragraphs) and provide sketches to support you definitions and descriptions for the following (10%)

  - Lobe Cams

Solution: Lobe cams come in many shapes and sizes and produced multiple events per cycle.  This lobe cam generates 4 events per cycle.  Each event is a steady upward motion followed by a sudden downward one.

  - Dwell (or Pause)

Solution: Dwell refers to radii on a cam that are raised and/or dipped.  At points where the profile returns to a constant radius, then there is no movement in the follower.

  - 3-bar linkages

Solution: 3-bar linkage is used for straight-line motion.  In the figure shown, pegs prevent Bar 1 from rotating too far with the crank.  Consequently, Bar 2 only moves side-to-side.  This finally results in Bar 3 approximately straight-line motion.

- Sketch and describe the 4 cycles of a 3-bar linkage.  In your sketch show where the straight line motion is and in your description, explain how the straight motion arises (10%)

Solution:

- Slider-crank makes top of Bar 1 trace an elliptical path (point A).

- Bar 2 is a lever with a fixed pivot at ground

- Bar 3 connected to Bars 1 and 2

- Top of Bar 3 (point B) approximates straight-line motion.

- An example of the common car windshield wiper (2 blades) is shown.  Before searching, think and sketch how you think the wiper works; what is the input and output?  Next, do a web search to study how wipers work.  Construct a Lego-based version using an NXT motor and NxC program.  Create Build Instructions as a 4-step build plan (one a single slide).  Capture a 10-second WMV video of your wiper in action (25%)

- Use MLCAD to create build instructions for the Lego-based folding chair assembly.  Recall, you constructed this assembly for a previous Homework assignment.  Bring a hardcopy of the build instructions to next week’s class.  Grading will depend on how easily your classmate can recreate your assembly using only the build instructions (25%)