Hands-on Lab

Lego Interfacing

Digital Input and Output (DIO) expanders, like the PCF8574, allow the NXT to interface to a wide range of devices.  The DIO lines can be used to actuate binary devices like switches, relays and transistors and 8-bit peripherals like external LCD screens.

Review – NXT and the PCF8574

The PCF8574 is an I2C chip that when connected to an NXT, gives the Brick 8 digital lines.  In previous concepts, these lines could turn on/off LEDs (output) or read a switch’s on/off position (input).  These concepts are the “Hello World” version of embedded micros.  The usefulness of such on/off output or input becomes clear when LEDs are replaced with relays or transistors.

Concept 1 – NXT and Reed Relays

A relay is an electromechanical switch.  A reed relay has a small form factor and is capable of handling low voltage, low current devices.  Energizing its coil (often labeled COM) will throw the relay’s switch from a normally open (often labeled N.O.) state to a closed one.

Step 1: With the PCF8574 circuit constructed from before, wire up the schematic (Fig. 1A).  In this circuit, only one digital (output) line (D0) is needed. Typically, a SPST (single pole, single throw) relay has 4 pins.  Two of them represent the coils’ ends.  The remaining two refer to the ends of switch.

Fig. 1A: Interfacing a reed relay to the NXT using one PCF8574 digital output line.

Fig. 1B: Part placement on a solderless breadboard (left).  The reed relay inserted into place (right)

The toy DC motor used in Fig. 1A can easily draw over 1 A.  Since the PCF8574 can only source about 20 mA, an external power supply is needed.  A 9V battery’s +’ve terminal connected to one end of the relay’s switch, and the –‘ve one connects to NXT’s GND pin.  This ties all ground lines together.

Step 2: Compose an NXC program called dioRelay1_0.nxc.

// FILE: dioRelay1_0.nxc - Works

// AUTH: P.Oh

// DATE: 09/19/16 11:26; 09/23/16 08:19

// VERS: 1.0 - PCF8574A digital output.  Reed relay (+5V coil) connected to D0

//             When D0 is LO, then relay COM should close

// NOTE: Uses PCF8574A chip (hence address A2-A1-A0 set to 0-0-0 hence 0x70

//       9V battery (or power supply) drives toy DC motor.

#define I2Cport S1 // Port number

#define I2CAddr8574 0x70 // I2C address x040 8574 or 0x70 for 8574A

task main() {

// array variables (since NXC's I2C functions take array variables

byte WriteBuf[2]; // data written to PCF8574A.  Declares a two one-byte variables

byte ReadBuf[]; // data received from PCF8574A.  We won't be reading any data but we need this for I2CBytes

int RdCnt = 1; // number of bytes to read

// button variables

bool orangeButtonPushed, rightArrowButtonPushed, leftArrowButtonPushed, greyButtonPushed, overflowFlag;

// Counting variables

int decimalNumber; // values from 0 to 255

SetSensorLowspeed (I2Cport); // PCF8574A connect to NXT on S1

// Prompt user to begin

// First, set address with first I2CWrite.  Recall, WriteBuf[1] has address 0xF0 0x00

WriteBuf[1] = 0x00; // i.e. write zeros to port sets up PCF8574A for writing

WriteBuf[0] = I2CAddr8574; // i.e. address is 0x70

I2CBytes(S1, WriteBuf, RdCnt, ReadBuf);

// Lets start with D0...D7 set to HI

WriteBuf[1] = 0xFF; // Port lines are HI; so, reed relay should be open and Line_Out should be LO

WriteBuf[0] = I2CAddr8574; // i.e. address is 0x70

I2CBytes(S1, WriteBuf, RdCnt, ReadBuf);

TextOut (0, LCD_LINE1, "Orange Btn starts");

do {

orangeButtonPushed = ButtonPressed(BTNCENTER, FALSE);

} while(!orangeButtonPushed);

ClearScreen();

TextOut(0, LCD_LINE1, "Orange BTN quits");

TextOut(0, LCD_LINE3, "Reed Relay");

TextOut(0, LCD_LINE4, "<-OFF/ON->");

decimalNumber = 0;

do {

greyButtonPushed = ButtonPressed(BTNEXIT, FALSE);

// If pressed, then grey button becomes TRUE.  If not pressed, then grey button is FALSE

rightArrowButtonPushed = ButtonPressed(BTNRIGHT, FALSE);

leftArrowButtonPushed = ButtonPressed(BTNLEFT, FALSE);

if(rightArrowButtonPushed) {

// right button pushed, so turn on relay's coils.

// set digital line D0 LO

decimalNumber = 0; // D0 is now LO; NXT's 5V can now flow thru coil, thus switch ON and motor ON

WriteBuf[1] = decimalNumber;

WriteBuf[0] = I2CAddr8574;

I2CBytes(S1, WriteBuf, RdCnt, ReadBuf);

TextOut (0, LCD_LINE6, FormatNum("%3d - Relay ON " , decimalNumber));

// now coils activated; switch is closed and resistance between COM and N.O. should be almost zero

};

if(leftArrowButtonPushed) {

// left button pushed, so turn off relay's coils

// set digital line D0 HI

decimalNumber = 1; // DO is now HI; NXT's 5V can't flow thru coil, thus switch remains closed and motor OFF

WriteBuf[1] = decimalNumber;

WriteBuf[0] = I2CAddr8574;

I2CBytes(S1, WriteBuf, RdCnt, ReadBuf);

TextOut (0, LCD_LINE6, FormatNum("%3d - Relay OFF" , decimalNumber));

// now coils are de-activated; switch is open and resistance between COM and N.O. should be infinite

};

Wait(250); // wait 250 millsec

} while(!greyButtonPushed && !overflowFlag);

TextOut(0, LCD_LINE5, "Finished!");

PlaySound(SOUND_DOUBLE_BEEP);

} // end main

Code Explanation: Much like dioOutput2_0.nxc from a previous lab, dioRelay1_0.nxc configures the PCF8574’s eight digital lines for output.  A do-while loop continuous reads button presses.  Pushing the right arrow button (rightArrowButtonPushed) closes the switch and turns the motor on and vice-versa with the left arrow button. The program aborts when the grey button is pressed.

Exercise 1: In NxC create programs for the following:

  - Replace the DC motor with a buzzer so that closing the relay’s switch results in a sound

  - Modify your code and wiring to turn the motor on using digital line D1.

Concept 2 – Concept 1 – NXT and Transistors

Like a relay, a transistor is a switch.  Unlike a relay, transistors do not have any mechanical parts.  As such, they are capable of switching much quicker than relays, and have longer mean-time-between-failures (MTBF).  Popular NPN transistors include the TIP31 and IRF510 which both come in TO-220 packages.  The TIP31 is a current-driven transistor, whereas the IRF510 is a voltage-driven one.  Since the PCF8574’s source current is low, turning on or off a current-hungry device like a motor is best done using the IRF510.  The IRF510 is called a MOSFET and allows current to flow when its gate pin is above a certain voltage (about 5 Volts).  One could employ a TIP31, but then a current-to-voltage converter would be needed.

Step 1: Breadboard the schematic in Fig. 2A, noting the IRF510’s pin labels (Fig. 2B)

Fig. 2A: Interfacing a IRF510 MOSFET to NXT using one of the PCF8574’s digital output lines.

Fig. 2B: The IRF510 MOSFET can simply insert into the solderless breadboard.

Fig. 2A does not use any diodes (and capacitors) in contrast to Fig. 1A.  While the circuit in Fig. 2A will work, the back-EMF from the motor could kick-back enough current to damage the digital line.  The analogy is back-wash in hydraulics.  A capacitor across the motor leads and a diode will prevent such back-wash to occur.

Step 2: Write and execute an NxC program called dioMosfet1_0.nxc

Much like dioOutput2_0.nxc and dioRelay1_0.nxc, one can set digital line D0 HI (+5V) or LO (GND) by setting decimalNumber to 1 or 0 respectively.

When the right arrow button is pressed, have the motor turn on and vice-versa with the left arrow button.  Use the grey button to abort the program.

Exercise 2:

2-1: Refer to dioInput2_0.nxc and dioDipLed2_0.nxc from a previous lab.  Write a program that reads DIP switch positions, and when the value is one, turn on the motor.  For any other DIP switch value, the motor turns off.  You can use either a reed relay or IRF510 circuit.