Supplemental Notes

Lego SCARA arm – Determining Vertical Resolution

Figure 1A: A 3-DOF SCARA arm constructed from Lego and XL-320 Dynamixels (left).  The vertical DOF (right) uses worm gears and a wedge belt pulley.

Figure 1A: Classic 3-DOF SCARA arm constructed with three XL-320 Dynamixels and Lego

Vertical Resolution

Question 1: How does the vertical DOF work?

Answer 1: First, observe in Figure 1A (right) that Lego part 4185 (wedge belt pulley) slides across Lego part 4716 (worm gear).  In other words, this is mechanism isn’t a traditional worm-spur gear.  Rather, the worm simply acts as a threaded “screw” and the wedge belt pulley acts as a “nut”.  As the worm rotates, the wedge translates.

Question 2: If the worm acts as a screw, what are its dimensions?

Figure 1B: Worm gear has 5 peak-to-peak spacings (yellow braces) and spans 2 studs

Answer 2: Observing the worm gear (Figure 1B left), one easily counts 5 peak-to-peak spacings.  This means it takes 5 worm gear revolutions to go from end-to-end.

Also, one sees (Figure 1B right), the worm gear is the same size as an Axle 2.  Thus the worm gear spans 2 studs.  Recall a stud, which also known as a Fundamental Lego Unit (FLU) is 8 millimeters.  Thus we can say the worm gear:

5 revolutions2 studs=2.5 revolutionsstud

Question 3: How many revolutions does it take to span one Brick height?

Figure 1C: From http://cs.wellesley.edu/~rds/handouts/LEGOStructures.pdf, all Lego bricks are 6/5 FLU (or studs) tall (left).  Since a FLU is 8 mm, this means the Brick is 9.6 mm tall (right).  Note: height does not include the stud part.

Vernier calipers indeed confirms 9.6 mm is the height of a Brick 1x1

Answer 3: Referring to Figure 1C, one calculates the following:

2.5 revolutionsFLU×65FLU=3 revolutions

A YouTube video demonstrates 3 revolutions results in 9.6 mm translation https://youtu.be/OvmJYHDu_bo using the NXC program xl320-scaraTest0_1b.nxc.

Question 4: What’s the total vertical distance this arm can travel?

Answer 4: The mechanism employs two worm gears.  This means 10 peak-to-peak spacings in total.  Hence, it will take 10 revolutions for the wedge belt pulley to translate 4 FLU = 4*8 millimeters = 32 millimeters.

Question 5: What’s the vertical resolution?  That is, how many millimeters of travel per degree?

Figure 1D: Source https://emanual.robotis.com/docs/en/dxl/x/xl320/ shows the XL-320 spans 0 to 300 degrees in servo mode.  This means the XL-320 has a 0.29 degree/count resolution.

Answer 5: Recall that the XL-320’s encoder spans from 0 to 1023 counts for motion of 0 to 300 degrees.  This results in 0.29 degrees/count.

One has:

1 revolution360 degrees×FLU2.5 revolutions×8 mmFLU=8.89×10-3mmdeg

Alternatively this means 112.5 degrees per millimeter of travel.

In terms of count:

8.89×10-3mmdeg×0.29degcount=2.31×10-3mmcount

Alternatively this means 432 counts per millimeter.